MACcess - EuroCrypt M smart card emulator

-----------------------------------------

NOTE: on receivers with a 4MHz clock freq. (ex: Pace Manhattan, Origo)

the baudrate is higher. 10000 baud has been tested and worked.

If changing the baudrate doesn't help and if you don't use an

external power supply for the interface and the program resets

all the time, then try using an ext. pow. sup.

Command line:

-------------

MAC [S|B] [com [baud [delay1 [delay2 [delay3 [delay4]]]]]]

S      - silent mode, disables 'OSD' (on computer)         (default: off )

B      - blank mode, turns screen black                    (default: off )

com    - comport 1 to 4                                    (default: 1   )

baud   - baudrate (if 9600 doesn't work try 10000)         (default: 9600)

delay1 - delay: reset released / send reset string         (default: 300 )

delay2 - delay: last data byte / status word(ex. 90 00)    (default: 0   )

delay3 - delay: between outgoing bytes                     (default: 100 )

delay4 - delay: between receiving data in a 88h ins.       (default: 300 )

ex: MAC S 2 10000 400 1500 1000 400

in the program 'delay1' is seen as 'AtoR' in the info window, 'delay2'

as 'SW', 'delay3' as 'send' and 'delay4' as '88h'. All delays are in

micro seconds. Currently because of a bug maximum is 65535 �s.

Keys:

-----

Note: The commands B, L, S (and quit of course) may be use at any time,

the rest should only be used when the program is not decrypting

(i.e Offline) else you'll get a "check your card" message.

'#' - Enter unique address, shared address (and position) and geographical

address (and theme code). Press ESC to skip an entry.

'1' to '4' - change delays 1 to 4 (as mentioned above maximum is 65535 �s).

ESC to abort.

'B' - toggle BLANK mode. Like SILENT mode except the screen is blank.

'C' - enter CI (in hexadecimal). The current value can be seen in the

help window. The EuroCrypt M standard value is 20h. ESC to abort.

'H' - help window, diplays all commands and some other info.

'L' - enables/disables EMM logging. Writes EMM's to "EMM.LOG". If the log

file allready exists the new data will be appended, this is in order

to prevent accidental overwriting of data.

'S' - toggle SILENT mode. When enabled no data will be printed (the info

window will still be updated).

'V' - enter the card number on a VIASAT/InterAccess card. The address of

these cards contains the shared address, this option makes it easier

for the user (since the shared address doesn't have to be entered

seperatly). ESC to abort.

'Q', 'X', ESC - quits the emulator.

Some notes:

-----------

Free EMM:s are displayed in normal colour, shared EMM:s are displayed in

red and unique EMM:s in light red.

The date in the info window is not the system date, it's the one

included in the data from the decoder. Some channels don't use this

so the date might be completely wrong.

The keys for Canal+ and CineCinemas are auto updated. If you haven't watched

Canal+/CineCinemas for a while it might take a moment before the picture

is descrambled (during this you might also get an OSD message "HASH ERROR",

nothing to worry about ... I hope).

Channels: FilmNet+, TV3, TV1000, FilmMax, Canal+, CineCinemas

(if FilmMax doesn't decode try restarting the prog.)

EMM.LOG:

--------

Besides the actual data the logfile contains info about when it was

started, ended, which card address and channel ID. Changes of

channel ID or card address are also logged (if logging is enabled).

Here are two data lines, with explaination, as an example:

Time      A4h-data     Index(header[3])

�

23:49:54 (00 04 01) F0 01: 9e 20 00 00 00 00 etc..............

23:49:54 (00 04 01) 01 01: a8 06 1d 01 23 12 etc..............

�      �

F0/00/01/02      Data begins here

F0 = f0h-message

00 = free/common 18h-mess. (EMM)

01 = shared      18h-mess. (EMM)

02 = unique      18h-mess. (EMM)