Pilot Assembler Reference

1st Release - April 29 97

Motorola 68000 Instruction Set

| Instructions | Description | Assembler Syntax | Data Size | C X | o N | d Z | e V | s C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Data Movement Operations

| EXG | Exchange any two registers | Rn,Rn |  |  | L | - | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LEA | Load Effective Address | <ea>,An |  |  | L | - | - | - | - | - |
| LINK | Allocate Stack Frame | An,#<displ> |  | W | L | - | - | - | - | - |
| MOVE | Between Effective Addresses | <ea>,<ea> | B | W | L | - | * | * | 0 | 0 |
| MOVEA | MOVE Address | <ea>,An |  | W | L | - | - | - | - | - |
| MOVEM | MOVE Multiple Registers | <reg list>,<ea> <ea>,<reg list> |  | W | L | - | - | - | - | - |
| MOVEP | MOVE Peripheral | Dn,x(An) x(An),Dn |  | W | L | - | - | - | - | - |
| MOVEQ | MOVE 8-bit immediate | #<-128 to +127>,Dn |  |  | L | - | * | * | 0 | 0 |
| PEA | Push Effective Address | <ea> |  |  | L | - | - | - | - | - |
| UNLK | Deallocate Stack Frame | An |  |  | L | - | - | - | - | - |

Integer Arithmetic Operations

| ADD | ADD binary | Dn,<ea> <ea>,Dn | B | W | L | * | * | * | * | * |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADDA | ADD binary to An | <ea>,An |  | W | L | - | - | - | - | - |
| ADDI | ADD Immediate | #x,<ea> | B | W | L | * | * | * | * | * |
| ADDQ | ADD 3-bit immediate | #<1-8>,<ea> | B | W | L | * | * | * | * | * |
| ADDX | ADD eXtended | Dn,Dn -(An),-(An) | B | W | L | * | * | * | * | * |
| CLR | CLeaR | <ea> | B | W | L | - | 0 | 1 | 0 | 0 |
| CMP | CoMPare | <ea>,Dn | B | W | L | - | * | * | * | * |
| CMPA | CoMPare Address | <ea>,An |  | W | L | - | * | * | * | * |
| CMPI | CoMPare Immediate | #<data>,<ea> | B | W | L | - | * | * | * | * |
| CMPM | CoMPare Memory | (An)+,(An)+ | B | W | L | - | * | * | * | * |
| DIVS | DIVide Signed | <ea>,Dn |  | W |  | - | * | * | * | 0 |
| DIVU | DIVide Unsigned | <ea>,Dn |  | W |  | - | * | * | * | 0 |
| EXT | Sign EXTend | Dn | B | W |  | - | * | * | 0 | 0 |
| MULS | MULtiply Signed | <ea>,Dn |  | W |  | - | * | * | 0 | 0 |
| MULU | MULtiply Unsigned | <ea>,Dn |  | W |  | - | * | * | 0 | 0 |
| NEG | NEGate | <ea> | B | W | L | * | * | * | * | * |
| NEGX | NEGate with eXtend | <ea> | B | W | L | * | * | * | * | * |
| SUB | SUBtract binary | Dn,<ea> <ea>,Dn | B | W | L | * | * | * | * | * |
| SUBA | SUBtract binary from An | <ea>,An |  | W | L | - | - | - | - | - |
| SUBI | SUBtract Immediate | #x,<ea> | B | W | L | * | * | * | * | * |
| SUBQ | SUBtract 3-bit immediate | #<1-8>,<ea> | B | W | L | * | * | * | * | * |
| SUBX | SUBtract eXtended | Dn,Dn -(An),-(An) | B | W | L | * | * | * | * | * |

Logical Operations

| AND | Bit-wise AND | <ea>,Dn Dn,<ea> | B | W | L | - | * | * | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ANDI | Bit-wise AND with Immediate | #<data>,<ea> | B | W | L | - | * | * | 0 | 0 |
| EOR | Exclusive OR | Dn,<ea> | B | W | L | - | * | * | 0 | 0 |
| EORI | Exclusive OR Immediate | #<data>,<ea> | B | W | L | - | * | * | 0 | 0 |
| NOT | Form one's complement | <ea> | B | W | L | - | * | * | 0 | 0 |
| OR | Bit-wise OR | <ea>,Dn Dn,<ea> | B | W | L | - | * | * | 0 | 0 |
| ORI | Bit-wise OR with Immediate | #<data>,<ea> | B | W | L | - | * | * | 0 | 0 |
| TST | TeST for negative or zero | <ea> | B | W | L | - | * | * | 0 | 0 |

Shift & Rotate Operations

| ASL | Arithmetic Shift Left | #<1-8>,Dn Dn,Dn <ea> | B B | W W W | L L | * | * | * | * | * |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASR | Arithmetic Shift Right | ... | B | W | L | * | * | * | * | * |
| LSL | Logical Shift Left | ... | B | W | L | * | * | * | 0 | * |
| LSR | Logical Shift Right | ... | B | W | L | * | * | * | 0 |  |
| ROL | ROtate Left | ... | B | W | L | - | * | * | 0 | * |
| ROR | ROtate Right | ... | B | W | L | - | * | * | 0 | * |
| ROXL | ROtate Left with eXtend | ... | B | W | L | * | * | * | 0 | * |
| ROXR | ROtate Right with eXtend | ... | B | W | L | * | * | * | 0 | * |
| SWAP | SWAP words of Dn | Dn |  | W |  | - | * | * | 0 | 0 |

Bit Manipulation Operations

| BCHG | Test a Bit and CHanGe | Dn,<ea> #<data>,<ea> | B |  | L | - | - | * | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCLR | Test a Bit and CLeaR | ... | B |  | L | - | - | * | - | - |
| BSET | Test a Bit and SET | ... | B |  | L | - | - | * | - | - |
| BTST | Bit TeST | ... | B |  | L | - | - | * | - | - |

BCD Operations

| ABCD | Add BCD with extend | Dn,Dn -(An),-(An) | B |  |  | * | U | * | U | * |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NBCD | Negate BCD | <ea> | B |  |  | * | U | * | U | * |
| SBCD | Subtract BCD with eXtend | Dn,Dn -(An),-(An) | B |  |  | * | U | * | U | * |

Program Control Operations

Conditional

| Bcc.S | Conditional Branch | <label> | B |  |  | - | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bcc.W | Conditional Branch | <label> |  | W |  | - | - | - | - | - |
| DBcc | Looping Instruction | Dn,<label> |  | W |  | - | - | - | - | - |
| Scc | Set to -1 if True, 0 if False | <ea> | B |  |  | - | - | - | - | - |

| N | Negative - Set if most significant bit of the result is set. Cleared otherwise |
| --- | --- |
| Z | Zero - Set if result equals zero. Cleared otherwise |
| V | Overflow - Set if there was an arithmetic overflow. This implies that the result is not representable in the operand size. Cleared otherwise |
| C | Carry - Set if carry is generated out of the most significant bitof the operands for an addition. Also, set if a borrow is generated in a subtraction. Cleared otherwise |
| X | Transparent to data movement. When affected, by arithmetic operations, it is set the same as the Carry bit. |

Condition Codes after CMP D0,D1

| Relation | Unsigned | Signed |
| --- | --- | --- |
| D1 < D0 | CS - Carry bit Set | LT - Less Than |
| D1 <= D0 | LS - Lower or Same | LE - Less than or Equal |
| D1 = D0 | EQ - EQual (Z-bit Set) | EQ - EQual (Z-bit Set) |
| D1 != D0 | NE - Not Equal (Z-bit Clear) | NE - Not Equal (Z-bit Clear) |
| D1 > D0 | HI - HIgher than | GT - Greater Than |
| D1 >= D0 | CC - Carry bit Clear | GE - Greater than or Equal |

| Bcc | T - Branch Always (use BRA) |
| --- | --- |
| Bcc | F - Never Branch (use NOP) |
| DBcc | F - Never Terminate (use DBRA) |
| DBcc | T - Always Terminate |
| Scc | SF - Never Set |
| Scc | ST - Always Set |

|  | Condition | Test (underline means NOT) |
| --- | --- | --- |
| T | True | 1 |
| F | False | 0 |
| HI | HIgh | C · Z |
| LS | Low or Same | C + Z |
| CC | Carry Clear | C |
| CS | Carry Set | C |
| NE | Not Equal | Z |
| EQ | EQual | Z |
| VC | oVerflow Clear | V |
| VS | oVerflow Set | V |
| PL | PLus | N |
| MI | MInus | N |
| GE | Greater or Equal | N · V + N · V |
| LT | Less Than | N · V + N · V |
| GT | Greater Than | N · V · Z + N · V · Z |
| LE | Less or Equal | Z + N · V + N · V |

Unconditional

| BRA.S | BRAnch | <label> | B |  |  | - | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BRA.W | BRAnch | <label> |  | W |  | - | - | - | - | - |
| BSR.S | Branch to SubRoutine | <label> | B |  |  | - | - | - | - | - |
| BSR.W | Branch to SubRoutine | <label> |  | W |  | - | - | - | - | - |
| JMP | JuMP to effective Address | <ea> |  |  |  | - | - | - | - | - |
| JSR | Jump to SubRoutine | <ea> |  |  |  | - | - | - | - | - |
| NOP | No OPeration | none |  |  |  | - | - | - | - | - |

Returns

| RTR | ReTurn and Restore | none |  |  |  | I | I | I | I | I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RTS | ReTurn from Subroutine | none |  |  |  | - | - | - | - | - |

System Control Operations

Privileged

| ANDI | Bit-wise AND Immediate with SR | #<data>,SR |  | W |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EORI | Exclusive OR Immediate with SR | #<data>,SR |  | W |  |  |  |  |  |  |
| MOVE | To SR | <ea>,SR |  | W |  | I | I | I | I | I |
| MOVE | From SR | SR,<ea> |  | W |  | - | - | - | - | - |
| MOVE | USP to/from Address Register | USP,An An,USP |  |  | L | - | - | - | - | - |
| ORI | Bit-wise OR immediate with SR | #<data>,SR |  | W |  |  |  |  |  |  |
| RESET | RESET all external devices | none |  |  |  | - | - | - | - | - |
| RTE | ReTurn from Exception | none |  |  |  | I | I | I | I | I |
| STOP | Enable & wait for interrupts | #<data> |  | W |  | I | I | I | I | I |

Trap Generating

| CHK | CHecK Dn Against Bounds | <ea>,Dn |  | W |  | - | * | U | U | U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ILLE GAL | ILLEGAL-Instruction Exception | none |  |  |  | - | - | - | - | - |
| TRAP | Execute TRAP Exception | #<vector> |  |  |  | - | - | - | - | - |
| TRAPV | TRAPV Exception if V-bit Set | none |  |  |  | - | - | - | - | - |

Condition Code Register

| ANDI | Bit-wise AND Immediate with CCR | #<data>,CCR | B |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EORI | Exclusive OR Immediate with CCR | #<data>,CCR | B |  |  |  |  |  |  |  |
| MOVE | Move to/from CCR | <ea>,CCR CCR,<ea> |  | W |  | I | I | I | I | I |
| ORI | Bit-wise OR immediate with CCR | #<data>,CCR | B |  |  |  |  |  |  |  |

Multiprocessor Operations

| TAS | Test & Set MSB & Set N/Z-bits | <ea> | B |  |  | - | * | * | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Symbol Meaning

| * | Set according to the result of the operation |
| --- | --- |
| - | Not affected by the operation |
| 0 | Cleared |
| 1 | Set |
| U | Undefined state after the operation |
| I | Set by immediate data |
| <ea> | Effective Address Operand |
| #<data> | Immediate data |
| <label> | Assembler label |
| <vector> | TRAP instruction Exception vector (0-15) |
| <reg lst> | MOVEM instruction register specification list |
| <displ.> | LINK instruction negative displacement |
| ... | Same as previous instruction |

Addressing Modes

| Data Register Direct | Dn |
| --- | --- |
| Address Register Direct | An |
| Address Register Indirect | (An) |
| Address Register Indirect with Post-Increment | (An)+ |
| Address Register Indirect with Pre-Decrement | -(An) |
| Address Register Indirect with Displacement | w(An) |
| Address Register Indirect with Index | b(An,Rx) |
| Absolute Short | w |
| Absolute Long | l |
| Program Counter with Displacement | w(PC) |
| Program Counter with Index | b(PC,Rx) |
| Immediate | #<data> |
| Status Register | SR |
| Condition Code Register | CCR |

Legend

| Dn | Data Register (n is 0-7) |
| --- | --- |
| An | Address Register (n is 0-6) |
| A7 | Stack Pointer (User or Supervisor) |
| b | 8 bit constant |
| w | 16 bit constant |
| l | 32 bit constant |
| x | 8, 16, 32 bit constant |
| Rn | Any address or data register |
| Rx | Index Register Specification, one of: |
| Dn.w | Low 16 bits of Data Register |
| Dn.l | All 32 bits of Data Register |
| An.w | Low 16 bits of Address Register |
| An.l | All 32 bits of Address Register |

Copilot Debugger Help

d	[<address>] [l<count>]	Dump memory using previous type

da	[<address>] [l<count>]	Dump memory ASCII

db	[<address>] [l<count>]	Dump memory bytes

dd	[<address>] [l<count>]	Dump memory double-words (longs)

dw	[<address>] [l<count>]	Dump memory words

dh	[-d] <handle>	Dump handle ptr, size (-d dumps data too)

ds		Dump active module's symbols

bc	<bpid>	Clear breakpoint

bd	<bpid>	Disable breakpoint

be	<bpid>	Enable breakpoint

bl		List breakpoint(s)

bp	[-na] [<address>]	Set breakpoint

(-na breaks at next app startup)

e	<address> <value> [...]	Enter using previous type

ea	<address> <value> [...]	Enter ASCII

eb	<address> <value> [...]	Enter bytes

ed	<address> <value> [...]	Enter double-words

ew	<address> <value> [...]	Enter words

g	[=<address>] [<address>]	Go and stop when addr is reached

*j	<expr> ?	"<true cmd list>" : "<false cmd list>"

k		Stack backtrace

ln	[<address>]	List closest symbols before and after addr

p	[=<address>] [<count>]	Execute an instruction

(won't trace into JSR, etc)

r	[<reg> <value>]	Dump or set registers

set	[<var> [<value>]]	Dump, clear, or set a variable's value

t	[=<address>] [<count>]	Trace an instruction

u	[<address>] [l<count>]	Display unassembled instructions

*zd	Run the default command

*zl	List the default command

*zs "<cmd list>"	Set the default command

n [<radix>]	Show/set default number base (radix)

?, help	Show this help

<Enter>	Repeat the previous command

| Legend |  |  |  |  | Pilot button |  | PC key |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <reg> |  |  |  | On/off |  | Esc |  |
| <address> |  |  |  | Date Book |  | F1 |  |
| <value> |  |  |  | Address |  | F2 |  |
| <range> | [<address>] [l<expr>] |  |  | To Do List |  | F3 |  |
| <expr> |  |  |  | Memo Pad |  | F4 |  |
| <bpid> | <id> ¦ '*' |  |  | Page Up |  | Up |  |
| <cmd list> |  |  |  | Page Down |  | Down |  |
| <var> |  |  |  |  |  |  |  |

Debugger Variables

_NumLines (0-any)	Default # of lines dumped by 'u', 'd', etc cmds

_BreakCmd ("cmd")	Cmmd exec after step, trace, or break into debugger

_NextDump

_NextDisasm

Pila Quick Reference

Pila [options] sourcefile

| Option | Description |
| --- | --- |
| -l | Generate a listing file. The listing file has same name as the source file suffixed with '.lis'. |
| -c | Show full constant expansions for DC directives. |
| -d | Generate a bunch of debug output. This option is of little use to most people. |
| -s | Generate Copilot-compatible symbols. |
| -r | Do not generate 'code' and 'data' resources. |
| -t TYPE | Set the output PRC database's type to the specified four characters |

| Specifier | Radix |
| --- | --- |
| none | Decimal |
| $ | Hexadecimal |
| % | Binary |
| @ | Octal |

Operators (sorted by order of precedence from highest to lowest)

| Operator | Syntax | Description |
| --- | --- | --- |
| . | structure.member | Structure member |
| - | - expression | Unary minus |
| ~~ | expression | One's complement |
| (, ) | ( expression ) | Parenthesized expression |
| << | expression << count | Shift left |
| >> | expression >> count | Shift right |
| ¦ | expression ¦ expression | Logical OR |
| & | expression & expression | Logical AND |
| * | expression * expression | Multiply |
| / | expression / expression | Divide |
| \ | expression \ expression | Modulo |
| + | expression + expression | Add |
| - | expression - expression | Subtract |

Directives:

| appl "applicationname", 'apid' |  |
| --- | --- |
| data |  |
| code |  |
| res 'type', id res 'type', id, "datafile" |  |
| align size |  |
| proc procname([argument][,argument]...) beginproc endproc |  |
| call procname([argument.size][,argument.size]...) |  |
| [name] dc.[ b ¦ w ¦ l ] initializer [, initializer]... |  |
| [name] dcb.[ b ¦ w ¦ l ] blocksize, initializer |  |
| [name] ds.[ b ¦ w ¦ l ] blocksize |  |
| name equ expression name equ 'string' |  |
| global globalname.size | referenced by a5 |
| local localname.size | referenced by a6 |
| include "includefile" |  |
| list expression | 0 (off) or 1 (on) |
| name reg registerlist name set expression name set 'string' |  |
| struct structname member.size [member.size] endstruct |  |
| systrap systrap([argument][,argument]...) | returns data in d0 and pointers in a0. May corrupt d1 and a1. |
| syslibtrap libtrap([argument][,argument]...) |  |
| end |  |

PilRC Quick Reference

pilrc [-L language] [-I include path] [-R resfile] [-q] file.rcp [output path]

where:

-L language: Generate resource files for target language.

-I include path: Search include path when looking for include or bitmap files. Multiple paths to search may be specified by repeating the -I option.

-R resfile: Output a .res file specifying all the resources emitted by pilrc. This file may be included in your PilA program.

-q: Less noisy output, for you minimalists.

File.rcp: Input file describing the resources to be emitted. Each resource is written as a seperate file in the [output path] directory. The output filename is constructed by appending the hexcode resource id to the four character resource type. For example, a FORM (tFRM) with formid of 15 would be written as tfrm000f.bin.output pathDirectory where .bin files should be generated.

Example:

pilrc -I c:\resources -L french -R myprogram.res    myprogram.rcp c:\output

include file sintax :<Symbol.i> equ <Value.n>

Types

| .i | identifier |
| --- | --- |
| .s | string |
| .ss | multi line string (concatenate with \ ) |
| .n | number or expression |
| .p | position coordinate as number, expression or keyword as below: |
| AUTO | Automatic width or height. The width/height of the item is computed based on the text in the item. Valid only for widths or heights of items. |
| CENTER | Centers the item either horizontally or vertically. Only valid for left or top coordinate of an item. |
| PREVLEFT [ + ¦ - n] | Previous items left coordinate |
| PREVRIGHT [ + ¦ - n] | Previous items right coordinate |
| PREVWIDTH [ + ¦ - n] | Previous items width |
| PREVTOP [ + ¦ - n] | Previous items top coordinate |
| PREVBOTTOM [ + ¦ - n] | Previous items bottom coordinate |
| PREVHEIGHT [ + ¦ - n] | Previous items height |

NOTE: AUTO and CENTER must stand alone and are not valid in arithmetic expressions

FORM (tFRM) sintax:

FORM ID <FormResourceId.n> AT (<Left.p> <Top.p> <Width.p> <Height.p>)

[FRAME]

[NOFRAME]

[MODAL]

[SAVEBEHIND]

[USABLE]

[HELPID <HelpId.n>]

[DEFAULTBTNID <BtnId.n>]

[MENUID <MenuId.n>]

BEGIN

<OBJECTS>

END

<OBJECTS>: one or more of:

| TITLE | <Title.s> |
| --- | --- |
| BUTTON | <Label.s> ID <Id.n> AT (<Left.p> <Top.p> <Width.p> <Height.p>) [USABLE] [NONUSABLE] [DISABLED] [LEFTANCHOR] [RIGHTANCHOR] [FRAME] [NOFRAME] [BOLDFRAME] [FONT <FontId.n>] |
| PUSH- BUTTON | <Label.s> ID <Id.n> AT (<Left.p> <Top.p> <Width.p> <Height.p>) [USABLE] [NONUSABLE] [DISABLED] [LEFTANCHOR] [RIGHTANCHOR] [FONT <FontId>] [GROUP <GroupId.n>] |
| CHECK- BOX | <Label.s> ID <Id.n> AT (<Left.p> <Top.p> <Width.p> <Height.p>) [USABLE] [NONUSABLE] [DISABLED] [LEFTANCHOR] [RIGHTANCHOR] [FONT <FontId>] [GROUP <GroupId.n>] [CHECKED] |
| POPUP- TRIGGER | <Label.s> ID <Id.n> AT (<Left.p> <Top.p> <Width.p> <Height.p>) [USABLE] [NONUSABLE] [DISABLED] [LEFTANCHOR] [RIGHTANCHOR] [FONT <FontId.n>] |
| SELECTOR- TRIGGER | <Label.s> ID <Id.n> AT (<Left.p> <Top.p> <Width.p> <Height.p>) [USABLE] [NONUSABLE] [DISABLED] [LEFTANCHOR] [RIGHTANCHOR] [FONT <FontId.n>] |
| REPEAT- BUTTON | <Label.s> ID <Id.n> AT (<Left.p> <Top.p> <Width.p> <Height.p>) [USABLE] [NONUSABLE] [DISABLED] [LEFTANCHOR] [RIGHTANCHOR] [FRAME] [NOFRAME] [BOLDFRAME] [FONT <FontId.n>] |
| LABEL | <Label.s> ID <Id.n> AT (<Left.p> <Top.p>)[USABLE] [NONUSABLE] [FONT <FontId.n>] |
| FIELD | ID <Id.n> AT (<Left.p> <Top.p> <Width.p> <Height.p>)[USABLE] [NONUSABLE] [DISABLED] [LEFTALIGN] [RIGHTALIGN] [FONT <FontId.n>] [EDITABLE] [NONEDITABLE] [UNDERLINED] [SINGLELINE] [MULTIPLELINES] [MAXCHARS <MaxChars.n>] MAXCHARS is required for FIELD to work properly |
| POPUP- LIST | ID<Id.n> <IdList.n> |
| LIST | <Item.s> <Item2.s>... ID <Id.n> AT (<Left.p> <Top.p> <Width.p> <Height.p>) [USABLE] [NONUSABLE] [DISABLED] [VISIBLEITEMS <NumVisItems.n>] [FONT <FontId.n>] |
| FORM- BITMAP | AT (<Left.p> <Top.p>) BITMAP <BitmapId.n> [NONUSABLE] Notes:The bitmap referenced by FORMBITMAP must appear as a seperate resource in the rcp file via the BITMAP command. |
| GADGET | ID <Id.n> AT (<Left.p> <Top.p> <Width.p> <Height.p>) [USABLE] [NONUSABE] |
| TABLE | ID <Id.n> AT (<Left.p> <Top.p> <Width.p> <Height.p>) ROWS <NumRows.n> COLUMNS <NumCols.n> COLUMNWIDTHS <Col1Width.n> <Col2Width.n.>... |
| GRAFFITI- STATE- INDICATOR | AT (<Left.p> <Top.p>) |

MENU (MBAR) sintax:

MENU ID <MenuResourceId>

BEGIN

<PULLDOWNS>

END

<PULLDOWNS>: one or more of:

PULLDOWN <PulldownTitle.s>

BEGIN

<MENUITEMS>

END

<MENUITEMS>: one or more of:

MENUITEM <MenuItem.s> <MenuItemId.n> [AccelChar.c]

MENUITEM SEPARATOR

ALERT (tALT) sintax:

ALERT ID <AlertResrouceId.n>

[HELPID <HelpId.n>]

[INFORMATION] [CONFIRMATION] [WARNING] [ERROR]

BEGIN

TITLE <Title.s>

MESSAGE <Message.ss>

BUTTONS <Button.s> <BUTTON.s>...

END

VERSION (tVER) sintax:

VERSION ID <VersionResourceId.n> <Version.s>

example: VERSION ID 1 "0.09"

STRING (tSTR) sintax:

STRING ID <StringResourceId.n> <String.ss>

APPLICATIONICONNAME (tAIN) sintax:

APPLICATIONICONNAME ID <AINResourceId.n> <ApplicationName.s>

APPLICATION (APPL) sintax:

APPLICATION ID <ApplResourceId.n> <APPL.s>

Note: <APPL.s> must be 4 characters long

ICON (tAIB) sintax:

ICON <IconFileName.s> (Converts a Microsoft Windows style monochrome bitmap to tAIB Pilot icon resources with ID 1000)

BITMAP (Tbmp) sintax:

BITMAP ID <BitmapResourceId.n> <BitmapFileName.s> (Converts Microsoft Windows style monochrome bitmaps to TBmp Pilot bitmap resources)

TRANSLATION

TRANSLATION <Language.s>

BEGIN

<STRINGTRANSLATIONS>

END

Where <STRINGTRANSLATINOS> is one or more of:

<Original.s> = <Translated.ss>

TIP: For long strings define a short keyword and then define both native and foriegn translations for it.

Character Set

0	Standard	1	Bold	2	Large

Fonts 0,1 & 2 upper set

|  | 8 | 9 | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  | ° | À | Ð | à | ð |
| 1 |  | ‘ | ¡ | ± | Á | Ñ | á | ñ |
| 2 | ‚ | ’ | ¢ | ² | Â | Ò | â | ò |
| 3 | ƒ | “ | £ | ³ | Ã | Ó | ã | ó |
| 4 | „ | ” | ¤ | ´ | Ä | Ô | ä | ô |
| 5 | … | • | ¥ | µ | Å | Õ | å | õ |
| 6 | † | – | ¦ | ¶ | Æ | Ö | æ | ö |
| 7 | ‡ | — | § | · | Ç | × | ç | ÷ |
| 8 | ˆ | ˜ | ¨ | ¸ | È | Ø | è | ø |
| 9 | ‰ | ™ | © | ¹ | É | Ù | é | ù |
| A | Š | š | ª | º | Ê | Ú | ê | ú |
| B | ‹ | › | « | » | Ë | Û | ë | û |
| C | Œ | œ | ¬ | ¼ | Ì | Ü | ì | ü |
| D |  |  |  | ½ | Í | Ý | í | ý |
| E |  |  | ® | ¾ | Î | Þ | î | þ |
| F |  | Ÿ | ¯ | ¿ | Ï | ß | ï | ÿ |

Font 3 - Symbols

| 03 |  | 07 |  | 0B |  | 0F |  (graffiti) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 04 |  | 08 |  | 0C |  | 10 |  (graffiti) |
| 05 |  | 09 | space | 0D | Caps (graffiti) | 11 | \ (graffiti) |
| 06 |  | 0A | space | 0E | num (graffiti) | 12 |  (graffiti) |

Font 4 - Symbols

01		02		03		04	i (upper form tab help index)

Font 5 - Scroll buttons

01		02		03	 (gray)	04	 (gray)

Font 6 - Calculator

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  | space | 0 |  |  |  |  |
| 1 |  |  |  | 1 |  |  |  |  |
| 2 |  |  |  | 2 |  |  |  |  |
| 3 |  |  |  | 3 |  |  |  |  |
| 4 |  |  |  | 4 |  |  |  |  |
| 5 |  |  |  | 5 |  |  |  |  |
| 6 |  |  |  | 6 |  |  |  |  |
| 7 |  |  |  | 7 |  |  |  |  |
| 8 |  |  |  | 8 |  |  |  |  |
| 9 | space |  |  | 9 |  |  |  |  |
| A | space |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |  |
| C |  |  | , |  |  |  |  |  |
| D |  |  | - |  |  |  |  |  |
| E |  |  | . |  |  |  |  |  |
| F |  |  | e |  |  |  |  |  |

Number Conversion

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 16 | 32 | 48 | 64 | 80 | 96 | 112 |
| 1 | 1 | 17 | 33 | 49 | 65 | 81 | 97 | 113 |
| 2 | 2 | 18 | 34 | 50 | 66 | 82 | 98 | 114 |
| 3 | 3 | 19 | 35 | 51 | 67 | 83 | 99 | 115 |
| 4 | 4 | 20 | 36 | 52 | 68 | 84 | 100 | 116 |
| 5 | 5 | 21 | 37 | 53 | 69 | 85 | 101 | 117 |
| 6 | 6 | 22 | 38 | 54 | 70 | 86 | 102 | 118 |
| 7 | 7 | 23 | 39 | 55 | 71 | 87 | 103 | 119 |
| 8 | 8 | 24 | 40 | 56 | 72 | 88 | 104 | 120 |
| 9 | 9 | 25 | 41 | 57 | 73 | 89 | 105 | 121 |
| A | 10 | 26 | 42 | 58 | 74 | 90 | 106 | 122 |
| B | 11 | 27 | 43 | 59 | 75 | 91 | 107 | 123 |
| C | 12 | 28 | 44 | 60 | 76 | 92 | 108 | 124 |
| D | 13 | 29 | 45 | 61 | 77 | 93 | 109 | 125 |
| E | 14 | 30 | 46 | 62 | 78 | 94 | 110 | 126 |
| F | 15 | 31 | 47 | 63 | 79 | 95 | 111 | 127 |

|  | 8 | 9 | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 128 | 144 | 160 | 176 | 192 | 208 | 224 | 240 |
| 1 | 129 | 145 | 161 | 177 | 193 | 209 | 225 | 241 |
| 2 | 130 | 146 | 162 | 178 | 194 | 210 | 226 | 242 |
| 3 | 131 | 147 | 163 | 179 | 195 | 211 | 227 | 243 |
| 4 | 132 | 148 | 164 | 180 | 196 | 212 | 228 | 244 |
| 5 | 133 | 149 | 165 | 181 | 197 | 213 | 229 | 245 |
| 6 | 134 | 150 | 166 | 182 | 198 | 214 | 230 | 246 |
| 7 | 135 | 151 | 167 | 183 | 199 | 215 | 231 | 247 |
| 8 | 136 | 152 | 168 | 184 | 200 | 216 | 232 | 248 |
| 9 | 137 | 153 | 169 | 185 | 201 | 217 | 233 | 249 |
| A | 138 | 154 | 170 | 186 | 202 | 218 | 234 | 250 |
| B | 139 | 155 | 171 | 187 | 203 | 219 | 235 | 251 |
| C | 140 | 156 | 172 | 188 | 204 | 220 | 236 | 252 |
| D | 141 | 157 | 173 | 189 | 205 | 221 | 237 | 253 |
| E | 142 | 158 | 174 | 190 | 206 | 222 | 238 | 254 |
| F | 143 | 159 | 175 | 191 | 207 | 223 | 239 | 255 |

Low Memory Space PalmOS vrs 1

| initStack.l | 0 |  |  |
| --- | --- | --- | --- |
| initPC.l | 4 |  |  |
| busErr.l | 8 |  |  |
| addressErr.l | C |  |  |
| illegalInstr.l | 10 |  |  |
| divideByZero.l | 14 |  |  |
| chk.l | 18 |  |  |
| trap.l | 1C |  |  |
| privilege.l | 20 |  |  |
| trace.l | 24 |  |  |
| aTrap.l | 28 |  |  |
| fTrap.l | 2C |  |  |
| reserved12.l | 30 |  |  |
| coproc.l | 34 |  |  |
| formatErr.l | 38 |  |  |
| unitializedInt.l | 3C |  |  |
| reserved.32 | 40 |  |  |
| spuriousInt.l | 60 |  |  |
| autoVec1.l | 64 |  |  |
| autoVec2.l | 68 |  |  |
| autoVec3.l | 6C |  |  |
| autoVec4.l | 70 |  |  |
| autoVec5.l | 74 |  |  |
| autoVec6.l | 78 |  |  |
| autoVec7.l | 7C |  |  |
| trap0.l | 80 |  |  |
| trap1.l | 84 |  |  |
| trap2.l | 88 |  |  |
| trap3.l | 8C |  |  |
| trap4.l | 90 |  |  |
| trap5.l | 94 |  |  |
| trap6.l | 98 |  |  |
| trap7.l | 9C |  |  |
| trap8.l | A0 |  |  |
| trap9.l | A4 |  |  |
| trapA.l | A8 |  |  |
| trapB.l | AC |  |  |
| trapC.l | B0 |  |  |
| trapD.l | B4 |  |  |
| trapE.l | B8 |  |  |
| trapF.l | BC |  |  |
| unassigned.64 | C0 | user interrupt vectors |  |
| memCardSlots.b | 100 |  |  |
| dbgWasEntered.b | 101 |  |  |
| memCardInfoP.l | 102 |  |  |
| memSemaphoreID.l | 106 |  |  |
| memDebugMode.w | 10A |  |  |
| dmOpenList.l | 10C |  |  |
| dbgInDebugger.b | 110 |  |  |
| dbgTracing.b | 111 |  |  |
| dbgGlobalsP.l | 112 |  |  |
| dbgSerGlobalsP.l | 116 |  |  |
| sysAppInfoP.l | 11A |  |  |
| sysKernelDataP.l | 11E |  |  |
| sysDispatchTableP.l | 122 |  |  |
| sysOwnerIDsInUse.l | 126 |  |  |
| sysAMXAppInfoP.l | 12A |  |  |
| sysClockFreq.l | 12E |  |  |
| sysHardKeyCreators.l | 132 |  |  |
| sysBatteryCheckTimer.w | 136 |  |  |
| sysBatteryMinThreshold.b | 138 |  |  |
| hwrBatteryLevel.b | 139 |  |  |
| sysNextBatteryAlertTimer.w | 13A |  |  |
| sysBatteryWarnThreshold.b | 13C |  |  |
| sysDispatchTableRev.b | 13D |  |  |
| sysDispatchTableSize.w | 13E |  |  |
| sysLibTableP.l | 140 |  |  |
| sysLibTableEntries.w | 144 |  |  |
| sysConsoleStackChunkP.l | 146 |  |  |
| sysUIShellAppInfoP.l | 14A |  |  |
| sysTimerID.l | 14E |  |  |
| sysAutoOffEvtTicks.l | 152 |  |  |
| sysAutoOffSeconds.w | 156 |  |  |
| sysRandomSeed.l | 158 |  |  |
| slkGlobalsP.l | 15C |  |  |
| serGlobalsP.l | 160 |  |  |
| scrGlobalsP.l | 164 |  |  |
| fplGlobalsP.l | 168 |  |  |
| penGlobalsP.l | 16C |  |  |
| sysEvtMgrGlobalsP.l | 170 |  |  |
| sndGlobalsP.l | 174 |  |  |
| timGlobalsP.l | 178 |  |  |
| almGlobalsP.l | 17C |  |  |
| ftrGlobalsP.l | 180 |  |  |
| grfGlobalsP.l | 184 |  |  |
| keyGlobalsP.l | 188 |  |  |
| activeWindow.l | 18C |  |  |
| displayWindow.l | 190 |  |  |
| drawWindow.l | 194 |  |  |
| firstWindow.l | 198 |  |  |
| exitWindowID.l | 19C |  |  |
| enterWindowID.l | 1A0 |  |  |
| exitedWindowID.l | 1A4 |  |  |
| gState.grafMode.b | 1A8 |  |  |
| gState.patternMode.b | 1A9 |  |  |
| gState.pattern.w | 1AA |  |  |
| gState.customPtn.8 | 1AC | 64 bits |  |
| gState.fontId.b | 1B4 |  |  |
| gState.padding.b | 1B5 |  |  |
| gState.font.l | 1B6 |  |  |
| gState.underlineMode.w | 1BA |  |  |
| eventQ.l | 1BC |  |  |
| eventQIndex.w | 1C0 |  |  |
| eventQLength.w | 1C2 |  |  |
| lastScreenX.w | 1C4 |  |  |
| lastScreenY.w | 1C6 |  |  |
| lastPenDown.w | 1C8 |  |  |
| needNullTickCount.l | 1CA |  |  |
| uiCurrentFontPtr.l | 1CE |  |  |
| uiFontTable.0.l | 1D2 | 10C462CA | standard |
| uiFontTable.1.l | 1D6 | 10C3253C | bold |
| uiFontTable.2.l | 1DA | 10C46CFE | large |
| uiFontTable.3.l | 1DE | 10C2279C | symbol |
| uiFontTable.4.l | 1E2 | 10C22998 | symbol 11 |
| uiFontTable.5.l | 1E6 | 10C22920 | symbol 7 |
| uiFontTable.6.l | 1EA | 10C22A2C | led |
| uiCurrentFontID.b | 1EE |  |  |
| padding.b | 1EF |  |  |
| currentForm.l | 1F0 |  |  |
| insPtIsEnabled.b | 1F4 |  |  |
| insPtOn.b | 1F5 |  |  |
| insPtLoc.x.w | 1F6 |  |  |
| insPtLoc.y.w | 1F8 |  |  |
| insPtHeight.w | 1FA |  |  |
| insPtLastTick.l | 1FC |  |  |
| insPtBitsBehind.l | 200 |  |  |
| clipboard.1.item.l | 204 |  |  |
| clipboard.1.length.w | 208 |  |  |
| clipboard.2.item.l | 20A |  |  |
| clipboard.2.length.w | 20E |  |  |
| clipboard.3.item.l | 210 |  |  |
| clipboard.3.length.w | 214 |  |  |
| uiCurrentMenu.l | 216 |  |  |
| undoGlobals.mode.w | 21A |  |  |
| undoGlobals.start.w | 21C |  |  |
| undoGlobals.end.w | 21E |  |  |
| undoGlobals.bufferLen.w | 220 |  |  |
| undoGlobals.buffer.l | 222 |  |  |
| gsiState.b | 226 |  |  |
| gsiIsEnabled.b | 227 |  |  |
| gsiLocation.x.w | 228 |  |  |
| gsiLocation.y.w | 22A |  |  |
| uiExtensionsP.l | 22C |  |  |
| curUIAppInfoP.l | 230 |  |  |
| nextUIAppDBID.l | 234 |  |  |
| nextUIAppCmd.w | 238 |  |  |
| nextUIAppCmdPBP.l | 23A |  |  |
| nextUIAppCardNo.b | 23E |  |  |
| hwrDataWELevel.b | 23F |  |  |
| hwrWakeUp.w | 240 |  |  |
| hwrCPUDutyCycle.b | 242 |  |  |
| hwrPenDown.b | 243 |  |  |
| hwrCurTicks.l | 244 |  |  |
| hwrTotalRAMSize.l | 248 |  |  |
| hwrDozeSubTicks.l | 24C |  |  |
| padGlobalsP.l | 250 |  |  |
| sysResetFlags.b | 254 |  |  |
| sysBatteryKind.b | 255 |  |  |
| memMinDynHeapFree.l | 256 |  |  |
| sysPrefFlags.w | 25A |  |  |
| sysGlobalsP.l | 25C |  |  |
| sysMiscFlags.w | 260 |  |  |

Main CPU Registers

| Byte | scr | 000 | System Control |
| --- | --- | --- | --- |
| Word | csAGroupBase | 100 | Chip Select Group A Base |
| Word | csBGroupBase | 102 | Chip Select Group B Base |
| Word | csCGroupBase | 104 | Chip Select Group C Base |
| Word | csDGroupBase | 106 | Chip Select Group D Base |
| Word | csAGroupMask | 108 | Chip Select Group A Mask |
| Word | csBGroupMask | 10A | Chip Select Group B Mask |
| Word | csCGroupMask | 10C | Chip Select Group C Mask |
| Word | csDGroupMask | 10E | Chip Select Group D Mask |
| Long | csASelect0 | 110 | Group A Chip Select 0 |
| Long | csASelect1 | 114 | Group A Chip Select 1 |
| Long | csASelect2 | 118 | Group A Chip Select 2 |
| Long | csASelect3 | 11C | Group A Chip Select 3 |
| Long | csBSelect0 | 120 | Group B Chip Select 0 |
| Long | csBSelect1 | 124 | Group B Chip Select 1 |
| Long | csBSelect2 | 128 | Group B Chip Select 2 |
| Long | csBSelect3 | 12C | Group B Chip Select 3 |
| Long | csCSelect0 | 130 | Group C Chip Select 0 |
| Long | csCSelect1 | 134 | Group C Chip Select 1 |
| Long | csCSelect2 | 138 | Group C Chip Select 2 |
| Long | csCSelect3 | 13C | Group C Chip Select 3 |
| Long | csDSelect0 | 140 | Group D Chip Select 0 |
| Long | csDSelect1 | 144 | Group D Chip Select 1 |
| Long | csDSelect2 | 148 | Group D Chip Select 2 |
| Long | csDSelect3 | 14C | Group D Chip Select 3 |
| Word | csDebug | 150 | Chip Select debug |
| Word | pllControl | 200 | PLL Control |
| Word | pllFreqSel | 202 | PLL Frequency Select |
| Word | pllTest | 204 | PLL Test |
| Byte | pwrControl | 207 | Power Control |
| Byte | intVector | 300 | Interrupt Vector |
| Word | intControl | 302 | Interrupt Control |
| Word | intMaskHi | 304 | Interrupt Mask /HIGH word |
| Word | intMaskLo | 306 | Interrupt Mask /LOW word |
| Word | intWakeupEnHi | 308 | Interrupt Wakeup Enable |
| Word | intWakeupEnLo | 30A | Interrupt Wakeup Enable |
| Word | intStatusHi | 30C | Interrupt Status /HIGH word |
| Word | intStatusLo | 30E | Interrupt Status /LOW word |
| Word | intPendingHi | 310 | Interrupt Pending |
| Word | intPendingLo | 312 | Interrupt Pending |
| Byte | portADir | 400 | Port A Direction |
| Byte | portAData | 401 | Port A Data |
| Byte | portASelect | 403 | Port A Select |
| Byte | portBDir | 408 | Port B Direction |
| Byte | portBData | 409 | Port B Data |
| Byte | portBSelect | 40B | Port B Select |
| Byte | portCDir | 410 | Port C Direction |
| Byte | portCData | 411 | Port C Data |
| Byte | portCSelect | 413 | Port C Select |
| Byte | portDDir | 418 | Port D Direction |
| Byte | portDData | 419 | Port D Data |
| Byte | portDPullupEn | 41A | Port D Pull-up Enable |
| Byte | portDPolarity | 41C | Port D Polarity |
| Byte | portDIntReqEn | 41D | Port D Interrupt Request Enable |
| Byte | portDIntEdge | 41F | Port D IRQ Edge |
| Byte | portEDir | 420 | Port E Direction |
| Byte | portEData | 421 | Port E Data |
| Byte | portESelect | 423 | Port E Select |
| Byte | portFDir | 428 | Port F Direction |
| Byte | portFData | 429 | Port F Data |
| Byte | portFPullupEn | 42A | Port F Pull-up Enable |
| Byte | portFSelect | 42B | Port F Select |
| Byte | portGDir | 430 | Port G Direction |
| Byte | portGData | 431 | Port G Data |
| Byte | portGPullupEn | 432 | Port G Pull-up Enable |
| Byte | portGSelect | 433 | Port G Select |
| Byte | portJDir | 438 | Port J Direction |
| Byte | portJData | 439 | Port J Data |
| Byte | portJSelect | 43B | Port J Select |
| Byte | portKDir | 440 | Port K Direction |
| Byte | portKData | 441 | Port K Data |
| Byte | portKPullupEn | 442 | Port K Pull-up Enable |
| Byte | portKSelect | 443 | Port K Select |
| Byte | portMDir | 448 | Port M Direction |
| Byte | portMData | 449 | Port M Data |
| Byte | portMPullupEn | 44A | Port M Pull-up Enable |
| Byte | portMSelect | 44B | Port M Select |
| Word | pwmControl | 500 | PWM Control |
| Word | pwmPeriod | 502 | PWM Period |
| Word | pwmWidth | 504 | PWM Width |
| Word | pwmCounter | 506 | PWM Counter |
| Word | tmr1Control | 600 | Timer 1 Control |
| Word | tmr1Prescaler | 602 | Timer 1 Prescaler |
| Word | tmr1Compare | 604 | Timer 1 Compare |
| Word | tmr1Capture | 606 | Timer 1 Capture |
| Word | tmr1Counter | 608 | Timer 1 Counter |
| Word | tmr1Status | 60A | Timer 1 Status |
| Word | tmr2Control | 60C | Timer 2 Control |
| Word | tmr2Prescaler | 60E | Timer 2 Prescaler |
| Word | tmr2Compare | 610 | Timer 2 Compare |
| Word | tmr2Capture | 612 | Timer 2 Capture |
| Word | tmr2Counter | 614 | Timer 2 Counter |
| Word | tmr2Status | 616 | Timer 2 Status |
| Word | wdControl | 618 | Watchdog Control |
| Word | wdReference | 61A | Watchdog Reference |
| Word | wdCounter | 61C | Watchdog Counter |
| Word | spiSlave | 700 | SPI Slave |
| Word | spiMasterData | 800 | SPI Master Data |
| Word | spiMasterControl | 802 | SPI Master Control |
| Word | uControl | 900 | Uart Control |
| Word | uBaud | 902 | Uart Baud Control |
| Word | uReceive | 904 | Uart Receive |
| Word | uTransmit | 906 | Uart Transmit |
| Word | uMisc | 908 | Uart Miscellaneous |
| Long | lcdStartAddr | A00 | Screen Starting Address |
| Byte | lcdPageWidth | A05 | Virtual Page Width |
| Word | lcdScreenWidth | A08 | Screen Width |
| Word | lcdScreenHeight | A0A | Screen Height |
| Word | lcdCursorXPos | A18 | Cursor X Position |
| Word | lcdCursorYPos | A1A: | Cursor Y Position |
| Word | lcdCursorWidthHeight | A1C | Cursor Width and Height |
| Byte | lcdBlinkControl | A1F | Blink Control |
| Byte | lcdPanelControl | A20 | Panel Interface Control |
| Byte | lcdPolarity | A21 | Polarity Config |
| Byte | lcdACDRate | A23 | ACD (M) Rate Control |
| Byte | lcdPixelClock | A25 | Pixel Clock Divider |
| Byte | lcdClockControl | A27 | Clocking Control |
| Byte | lcdLastBufferAddr | A29 | Last Buffer Address |
| Byte | lcdOctetTermCount | A2B | Octet Terminal Count |
| Byte | lcdPanningOffset | A2D | Panning Offset |
| Byte | lcdFrameRate | A31 | Frame Rate Control Modulation |
| Word | lcdGrayPalette | A32 | Gray Palette Mapping |
| Byte | lcdReserved | A34 | Reserved |
| Long | rtcHourMinSec | B00 | RTC Hours, Minutes, Seconds |
| Long | rtcAlarm | B04 | RTC Alarm |
| Long | rtcReserved | B08 | RTC Reserved |
| Word | rtcControl | B0C | RTC Control |
| Word | rtcIntStatus | B0E | RTC Interrupt Status |
| Word | rtcIntEnable | B10 | RTC Interrupt Enable |
| Word | stopWatch | B12 | Stopwatch Minutes |