This disk contains a Serial Loader Program which is intended for use

in a DS5000, DS2250 or related product. Included is the source

assembly code, program listing, hex download files, and this doc

file. The code is intended for use by the customer, and is designed

to be adapted or modified to fit the application.

As shown in the DS5000 user's guide, application programs may

download new code, in effect re-programming themselves. This program

is designed to assist with the procedure. When through the course of

normal operations, or a new stimulus, the application program

determines that new code must be downloaded, a serial loader can be

invoked.

Using a terminal emulator, commands may be sent from a PC.

As this serial loader is designed, an ASCII  ^C  received on the

serial port will invoke the loader. Naturally, an RS232 interface

will be necessary. The stimuli which invokes the loader may be

modified for the end application. It may desirable to eliminate the

user interface and automatically load the RAM when the loader is

invoked. This would be similar to booting a system

When the loader is invoked, scratch pad registers are cleared. The

partition access bit is enabled. This action has the effect of

lowering the partition to 800H (2K). Therefore serial loader code

must be located below this mark and applications to be reloaded must

be above it. Upon entering the loader, a prompt is returned to the

user.  At this time the following commands may be executed.

L Load an Intel Hex format file.

D Dump an Intel hex format file from DS5000 RAM

V Verify an Intel hex file from DS5000 RAM

T Toggle echo feature

P Program the partition nibble

If the user desires to load a new program, type L, then use the

terminal to send the new file. Addresses above 800H may be modified.

If the new code is larger than the previous code, the new partition

may be modified by using P, with the partition register nibble.

If the program does not meet the exact application needs, it may be

freely modified or added to. It is meant as a framework to illustrate

serial downloading by the application without entering program load

mode. Comments in the source code should be helpful in determining

how the loader works. In the event that major modifications are

necessary, these should illustrate the program flow.

Note that in a secure application, this code provides a means of

downloading software without unlocking the processor. The dump

and trace commands should be disabled to prevent a back door entry.